Distributed Road Hazard Intelligence & Public Accountability System

1. Project Overview

Working Concept

A software platform that uses vehicle dashcam footage as a distributed sensing network to detect and map road hazards—primarily potholes—and make road-condition information publicly visible.

Instead of depending only on government road surveys, dedicated road-monitoring vehicles, or sensors installed on major highways, the system collects road-condition data from ordinary vehicles travelling across different types of roads.

The platform processes dashcam footage, identifies road hazards, determines their geographic location, assigns a severity/confidence score, and displays them on an interactive map.

The public can therefore see:

- Where potholes and other road hazards exist

- How severe they are

- Which roads are affected

- Which routes are relatively safer/better maintained

- How recently a hazard was detected

- Whether authorities have acknowledged or resolved the issue

A government/authority dashboard can subsequently use the same data to:

- Identify high-priority roads

- Track unresolved hazards

- Monitor recurring problem locations

- Assign maintenance priorities

- Record action taken

- Demonstrate improvement over time

2. Core Problem

Road hazards such as potholes create:

- Vehicle damage

- Two-wheeler accidents

- Traffic congestion

- Reduced road safety

- Increased travel time

- Poor emergency response

- Public dissatisfaction

The major problem is not only that potholes exist.

The deeper problem is:

There is no sufficiently continuous, distributed and transparent system for discovering, locating, prioritising and publicly tracking road hazards across all types of roads.

Traditional road monitoring can be:

- Periodic rather than continuous

- Limited by manpower

- Focused on major roads

- Expensive to scale

- Slow to reflect newly formed hazards

- Difficult for citizens to independently verify

3. Core Insight

Vehicles can become distributed road sensors.

A vehicle already travels through roads every day.

A dashcam already observes the road.

Therefore, instead of deploying expensive dedicated infrastructure everywhere:

Vehicle → Dashcam → Road footage → AI detection → Location → Hazard database → Public map

This creates a continuously growing road-condition dataset.

The system does not require every vehicle to participate.

Even a relatively small number of participating vehicles can gradually build a road-condition map.

4. Primary Objective

The primary objective is:

To create a scalable platform that continuously collects road-condition observations from vehicles, detects and geolocates road hazards, and converts them into publicly accessible and actionable road-safety intelligence.

The MVP should prove this complete chain:

Capture → Detect → Locate → Store → Visualise

Everything else should come after this.

5. What We Are Actually Building

The project should be considered as three connected systems.

A. Vehicle / Data Collection System

Responsible for obtaining road footage and associated location information.

Possible inputs:

- Dashcam video

- Smartphone camera

- GPS coordinates

- Timestamp

- Vehicle movement information

For the hackathon, this can initially be represented by:

- Uploaded dashcam footage

- A sample recorded route

- Simulated/recorded GPS metadata

The important thing is proving the pipeline rather than building sophisticated hardware.

B. Road Intelligence Backend

The backend is the core of the system.

It receives observations and performs:

- Video processing

- Hazard detection

- Location association

- Severity estimation

- Confidence calculation

- Duplicate detection

- Hazard storage

- Road-condition aggregation

This becomes the system's road intelligence layer.

C. Public + Authority Web Application

The frontend has two major experiences.

Public

Users can:

- View the road map

- See potholes/hazards

- Check severity

- Explore dangerous roads

- View route conditions

- Report/verify hazards

- Track whether issues have been resolved

Government / Authority

Authorities can:

- View detected hazards

- Filter by severity

- Identify high-risk areas

- View unresolved hazards

- Track issue status

- Assign maintenance actions

- Update resolution status

- Analyse road-condition trends

6. The Absolute MVP

For the hackathon, do not attempt to build everything above.

The MVP should answer one question:

Can we take vehicle footage, automatically identify road hazards, put them on a map at the correct locations, and use that information to show road risk?

The MVP should contain exactly these core components:

1. Dashcam/video input

Upload or process a recorded dashcam video.

2. Pothole detection

Computer vision model identifies potholes.

Example output:

Pothole detected

Confidence: 91%

Estimated severity: High

Frame: 183

3. Geolocation

Associate the detection with:

Latitude

Longitude

Timestamp

Road/route

4. Central database

Store every validated observation.

Example:

Hazard ID

Latitude

Longitude

Severity

Confidence

Detected At

Source

Status

5. Interactive map

Display hazards geographically.

For example:

- Green → low-risk road

- Yellow → moderate risk

- Red → high-risk road

- Hazard marker → specific pothole

6. Route condition

When a user selects a route, calculate a simple road-condition score.

Example:

Route A

Distance: 8.2 km

Hazards: 13

High severity: 3

Road Condition: 62/100

Risk: Moderate

7. Authority dashboard

Show:

Total hazards

High-severity hazards

Unresolved hazards

Most affected roads

Recent detections

Resolved hazards

8. Public accountability

Every hazard should have a lifecycle:

Detected

↓

Verified

↓

Reported to Authority

↓

Acknowledged

↓

Under Repair

↓

Resolved

For the hackathon, some of these transitions can be simulated.

7. The Most Important User Flow

The entire demonstration should revolve around one extremely clear flow.

Step 1 — Vehicle travels

A dashcam records:

Road footage

+

GPS

+

Timestamp

Step 2 — System processes footage

AI detects:

Pothole #001

Confidence: 94%

Severity: High

Step 3 — System determines location

12.9716° N

77.5946° E

Step 4 — Backend stores it

The hazard becomes a permanent road-condition observation.

Step 5 — Public map updates

The location appears on the map.

Step 6 — Route risk changes

A route passing through that location receives a worse road-condition score.

Step 7 — Authority sees it

The same hazard appears in the authority dashboard.

Step 8 — Accountability

The authority can update:

Status: Under Repair

and eventually:

Status: Resolved

That is the complete product story.

8. Map Experience

The map should be one of the strongest parts of the product.

It should feel familiar to users of Google Maps, but it should answer a different question:

"What is the condition of this road?"

Rather than only:

"How do I get there?"

Road Risk Visualization

Road segments can be represented using condition scores.

Example:

| Score | Condition |
| --- | --- |
| 80–100 | Good |
| 60–79 | Moderate |
| 40–59 | Poor |
| 0–39 | High Risk |

The actual scoring formula can evolve later.

For MVP:

Road Score =

100

- severity penalties

- hazard density penalties

The system does not need a scientifically perfect formula during the hackathon.

It needs a transparent and explainable one.

9. Hazard Model

Initially, focus on potholes.

Do NOT attempt to detect every possible road problem.

Future hazard classes could include:

- Potholes

- Cracks

- Road debris

- Broken road surfaces

- Waterlogging

- Missing manhole covers

- Damaged signs

- Fallen objects

- Road obstructions

- Poor lane markings

- Damaged speed breakers

- Unlit hazards

But the first version should be:

Pothole detection done well.

10. Severity Classification

A pothole should not simply be:

Pothole = Yes

The system should attempt to estimate severity.

Example:

Low

Small surface defect.

Medium

Potentially dangerous for two-wheelers.

High

Large/deep pothole capable of causing significant vehicle instability or damage.

For MVP, severity can initially be based on:

- Estimated size

- Visual characteristics

- Detection confidence

- Repeated observations

Later it can incorporate:

- Vehicle speed

- Camera perspective

- Depth estimation

- Suspension/IMU data

- Crowd confirmation

11. Confidence vs Severity

These must be kept separate.

Confidence

How confident the AI is that the object is a pothole.

Example:

Pothole probability = 94%

Severity

How dangerous the pothole appears.

Example:

Severity = High

A system could therefore produce:

Confidence: 96%

Severity: Medium

or:

Confidence: 81%

Severity: High

This distinction makes the system considerably more credible.

12. Multiple Vehicle Observations

One of the most important future capabilities is crowdsourced verification.

Suppose:

Vehicle A → detects pothole

Vehicle B → detects same pothole

Vehicle C → detects same pothole

The backend should recognise that these are probably the same physical hazard.

Instead of creating three potholes:

Pothole #145

Observed by:

3 vehicles

Confidence:

Very High

First detected:

10 Aug

Last detected:

22 Aug

Status:

Unresolved

This makes the dataset much more reliable.

13. Duplicate Detection

This should eventually become a backend capability.

If two detections occur within a geographic radius:

Detection A

Lat: X

Lon: Y

Detection B

Lat: X + small difference

Lon: Y + small difference

the system can determine that they refer to the same hazard.

Possible future logic:

Spatial distance

+

Time difference

+

Visual similarity

=

Same hazard probability

For MVP, a simple geographic threshold is sufficient.

14. Public Application

The public interface should focus on clarity rather than excessive features.

Main screen

Large interactive map.

Users see:

- Road-condition overlays

- Pothole markers

- Severity indicators

- Current location

- Search

- Route selection

Hazard details

Clicking a pothole should show:

Pothole #145

Severity: High

Confidence: 94%

First detected:

18 Aug 2026

Last observed:

22 Aug 2026

Observations:

7

Status:

Unresolved

15. Route Analysis

This is one of the most valuable user-facing features.

A user enters:

From: A

To: B

The system calculates route alternatives.

Instead of only saying:

Route A: 25 min

Route B: 29 min

your system can show:

Route A

25 min

Road condition: 54/100

Hazards: 12

High severity: 4

Route B

29 min

Road condition: 81/100

Hazards: 3

High severity: 0

This gives users an additional dimension of decision-making:

Time vs Road Safety/Condition

For the first MVP, route generation can be simplified or integrated with an existing mapping/routing provider.

16. Government / Authority Dashboard

The authority dashboard is NOT another generic admin panel.

Its purpose is:

Turning crowdsourced observations into maintenance priorities.

Dashboard Overview

Example:

Total Active Hazards       1,284

High Severity                186

Unresolved                    913

Under Repair                  147

Resolved                      224

Priority Areas

The system should identify:

Top affected roads

Top affected wards

Highest hazard density

Oldest unresolved hazards

Repeatedly reported hazards

17. Accountability Layer

This is what can make the project much more impactful.

Each hazard should have:

Detected

↓

Reported

↓

Acknowledged

↓

Assigned

↓

Repair in Progress

↓

Resolved

The system should preserve timestamps.

Example:

Detected:

10 Aug

Reported:

10 Aug

Acknowledged:

12 Aug

Repair Started:

16 Aug

Resolved:

19 Aug

This creates a basic public maintenance trail.

The system should avoid making unsupported accusations such as:

"Government failed to fix this."

Instead, it should present factual status information:

Unresolved for 12 days

That distinction matters for credibility.

18. Public Transparency

A user should be able to see:

Hazard detected 12 days ago

7 independent observations

Currently unresolved

This creates transparency without requiring the platform to make political or legal claims.

19. Data Architecture

A simplified architecture:

VEHICLE

│

▼

Dashcam Camera

│

▼

Video + GPS

│

▼

Processing Layer

│

┌──────┴──────┐

▼             ▼

Object Detection   GPS Processing

│             │

└──────┬──────┘

▼

Hazard Engine

│

┌───────────┼───────────┐

▼           ▼           ▼

Severity     Confidence   Deduplication

│           │           │

└───────────┼───────────┘

▼

Database

│

┌─────────┴─────────┐

▼                   ▼

Public Application   Authority Dashboard

│                   │

▼                   ▼

Road Map          Action / Status

20. Suggested Backend Entities

The database should eventually contain entities such as:

Users

id

name

email

role

created_at

Roles could include:

Public

Authority

Administrator

Vehicles / Sources

id

user_id

source_type

created_at

Trips

id

vehicle_id

start_time

end_time

route

Observations

Raw AI detections:

id

trip_id

latitude

longitude

timestamp

detection_confidence

severity

image/frame reference

Hazards

A consolidated physical road hazard:

id

latitude

longitude

hazard_type

severity

confidence

first_detected

last_detected

observation_count

status

Status History

id

hazard_id

status

changed_by

changed_at

This allows the accountability timeline to be reconstructed.

21. AI / Computer Vision Pipeline

The initial AI pipeline should be simple.

Video

↓

Frame extraction

↓

Pothole detection model

↓

Bounding box

↓

Confidence

↓

Frame timestamp

↓

GPS association

↓

Hazard record

Potential technologies can include:

- YOLO-based object detection

- OpenCV

- Python

- PyTorch

- ONNX

- Cloud inference or local inference

The exact model should be chosen based on accuracy, inference speed and availability of training data, not because it sounds impressive.

22. Geolocation Challenge

One of the hardest technical problems is:

How do we know exactly where the pothole in the video is?

A dashcam frame itself does not necessarily contain GPS information.

Possible solutions:

Option 1 — GPS-equipped dashcam

Best real-world approach.

Option 2 — Smartphone GPS

Phone provides location while camera captures footage.

Option 3 — GPX/video timestamp synchronization

Synchronize:

Video timestamp

+

GPS timestamp

Option 4 — Demo simulation

For hackathon:

Use a recorded route with associated GPS points and demonstrate the complete pipeline.

Do not pretend simulated GPS is real-time production infrastructure.

23. Privacy and Security

This must be considered from the beginning.

Dashcam footage may contain:

- Faces

- Vehicle registration plates

- Pedestrians

- Private locations

The platform should therefore avoid exposing raw footage publicly.

Possible privacy pipeline:

Raw video

↓

AI processing

↓

Blur faces / number plates

↓

Store only required evidence

↓

Public map

For the MVP, the public application should primarily expose:

- Hazard location

- Hazard type

- Severity

- Detection date

- Status

rather than entire raw videos.

24. What Makes This Different

The differentiation should NOT be:

"We use AI to detect potholes."

That alone is not sufficiently differentiated.

The stronger proposition is:

A distributed road-condition intelligence network powered by vehicles already travelling across the road network.

Your differentiation comes from combining:

Distributed collection

Vehicles become mobile sensors.

Coverage

Not restricted to major highways.

Continuous observations

Road conditions can change daily.

Public visibility

The information is accessible to citizens.

Authority workflow

Authorities can act on the same dataset.

Accountability

Resolution status and timelines can be tracked.

Route intelligence

Users can consider road condition while choosing routes.

25. What We Should NOT Build Initially

This is extremely important.

Do NOT spend the first development phase on:

- Mobile app for Android + iOS

- Advanced user profiles

- Social networking

- Complex gamification

- Payment system

- Full government authentication infrastructure

- IoT hardware

- Custom dashcam manufacturing

- Every possible road hazard

- Perfect nationwide map

- Complex AI model training from scratch

- Blockchain

- Cryptocurrency/rewards

- Sophisticated predictive maintenance

- Huge analytics suite

These can all come later.

26. MVP Priority Stack

P0 — Absolutely Required

These define whether the project works.

- Dashcam/video input

- Pothole detection

- GPS/location association

- Backend API

- Database

- Hazard records

- Interactive map

- Road-condition visualization

- Basic route condition

- Authority dashboard

- Hazard status

P1 — Add if Core System Works

- Multiple observations per hazard

- Duplicate detection

- Public hazard reporting

- Hazard verification

- Severity refinement

- Historical timeline

- Better route scoring

- Basic analytics

P2 — Future Product Features

- Live dashcam streaming

- Smartphone application

- Automatic GPS synchronization

- More hazard categories

- Road deterioration prediction

- Maintenance priority prediction

- Citizen reputation/verification

- Vehicle fleet integrations

- Government API integration

- Automated authority notifications

- Predictive road maintenance

- Regional/national road-condition intelligence

27. Hackathon Demonstration Scenario

The demo should tell a story rather than simply showing screens.

Scene 1 — The problem

Show a normal road.

Explain:

Thousands of vehicles travel through roads every day, but most of those vehicles are not contributing any structured information about road conditions.

Scene 2 — The vehicle

Show dashcam footage.

Vehicle travelling

↓

Dashcam captures road

↓

GPS records location

Scene 3 — Detection

Show AI detecting a pothole.

POTHOLE DETECTED

Confidence: 94%

Severity: HIGH

Scene 4 — Map

The detected pothole appears at its geographic location.

Then demonstrate several detections along the route.

The road gradually changes:

GOOD ───── MODERATE ───── POOR ───── HIGH RISK

Scene 5 — Public User

A user selects:

A → B

The system presents route condition.

Example:

Fastest Route

25 min

Condition: Poor

Alternative

29 min

Condition: Good

Scene 6 — Government

Switch to authority dashboard.

Show:

High Priority Road

8 active hazards

5 high severity

23 observations

12 days unresolved

Scene 7 — Accountability

Change:

Unresolved

to:

Under Repair

Then:

Resolved

Show the timeline.

28. The Core Product Loop

The entire platform can ultimately be reduced to this loop:

DRIVE

↓

CAPTURE

↓

DETECT

↓

LOCATE

↓

VERIFY

↓

MAP

↓

PRIORITIZE

↓

ACT

↓

VERIFY AGAIN

↓

RESOLVE

The final verify again step is particularly powerful.

If another vehicle later drives over the same road and the pothole is no longer detected:

Before:

High severity

After:

No hazard detected

the system has evidence that the road condition changed.

That turns the platform from a reporting system into a road-condition monitoring system.

29. Long-Term Vision

The long-term system is not simply a pothole map.

It can become:

A real-time digital layer representing the physical condition of a city's roads.

Every participating vehicle contributes observations.

Over time, the platform can understand:

Which roads deteriorate fastest?

Which areas have recurring problems?

Which hazards remain unresolved longest?

Which roads are improving?

Where should maintenance happen first?

Eventually:

Current Road Condition

+

Historical Data

+

Traffic

+

Weather

+

Maintenance History

↓

Predictive Road Intelligence

This could support proactive maintenance rather than waiting for accidents or complaints.

30. Product Success Metrics

The MVP should be evaluated using measurable outcomes.

Detection

- Pothole detection accuracy

- False positive rate

- Detection confidence

Coverage

- Road kilometres monitored

- Number of routes observed

- Number of unique road segments

Data quality

- GPS accuracy

- Duplicate detection accuracy

- Number of independent observations

Public usefulness

- Route condition availability

- Hazard visibility

- Route comparison

Authority usefulness

- Number of high-priority hazards identified

- Average unresolved duration

- Resolution tracking

31. Technical Success Criterion for the Hackathon

The project is successful if you can demonstrate:

A vehicle travels → dashcam footage is processed → a pothole is detected → its location is determined → it appears on the map → the road's risk score changes → an authority can see and manage the issue.

If this works reliably, stop adding features.

A polished complete pipeline is more valuable than 20 partially implemented features.

32. Proposed Technology Structure

A practical initial stack could be:

Frontend

React

+

Map library

Possible mapping options:

- Mapbox

- MapLibre

- OpenStreetMap-based services

- Google Maps where licensing/API requirements make sense

Avoid assuming that Google Maps itself is necessary simply because the interface resembles Google Maps.

Backend

Python

FastAPI

Responsibilities:

- API

- AI inference integration

- hazard processing

- route scoring

- authentication

- authority operations

Database

PostgreSQL

+

PostGIS

PostGIS is particularly valuable because this is fundamentally a geospatial application.

It allows the backend to work with:

- Points

- Roads

- Distances

- Geographic proximity

- Route segments

- Spatial queries

AI

YOLO / equivalent detection model

+

OpenCV

+

Python

Storage

Object storage for:

- Selected evidence frames

- Processed images

- Video files when required

Do not store enormous amounts of raw video indefinitely unless there is a specific reason.

33. Recommended System Architecture

┌──────────────────┐

│     VEHICLES     │

│     DASHCAMS     │

└────────┬─────────┘

│

Video + GPS

│

▼

┌─────────────────────┐

│  INGESTION SERVICE  │

└──────────┬──────────┘

│

▼

┌─────────────────────┐

│   AI PROCESSING     │

│                     │

│ Pothole Detection   │

│ Severity            │

│ Confidence          │

└──────────┬──────────┘

│

▼

┌─────────────────────┐

│ GEO / HAZARD ENGINE │

│                     │

│ Location            │

│ Deduplication       │

│ Aggregation         │

└──────────┬──────────┘

│

▼

┌─────────────────────┐

│ PostgreSQL/PostGIS  │

└──────────┬──────────┘

│

┌──────────────┴──────────────┐

│                             │

▼                             ▼

┌──────────────────┐         ┌──────────────────┐

│  PUBLIC WEB APP  │         │ AUTHORITY SYSTEM │

│                  │         │                  │

│ Map              │         │ Dashboard        │

│ Routes           │         │ Priorities       │

│ Hazards          │         │ Status           │

│ Reports          │         │ Analytics        │

└──────────────────┘         └──────────────────┘

34. Development Order

The team should build in this order.

Phase 0 — Define the data contract

Before UI.

Decide:

What is a hazard?

What fields does it contain?

What is a road segment?

What is an observation?

What is severity?

What is confidence?

What are the statuses?

Phase 1 — Build the backend

Create:

- Database

- Hazard model

- API

- Observation ingestion

- Basic geospatial queries

Phase 2 — Build AI pipeline

Get:

Video

→ Detection

→ JSON result

working independently.

Phase 3 — Connect AI + Backend

AI result

→ API

→ Database

Phase 4 — Build map

Database:

Hazards

↓

Frontend:

Map markers

+

road overlays

Phase 5 — Build route intelligence

Add:

Route

→ intersect hazards

→ calculate condition

→ display score

Phase 6 — Build authority dashboard

Add:

- Statistics

- Filters

- Hazard list

- Status changes

- Timeline

Phase 7 — Polish

Only after everything works:

- Animations

- Better UI

- Loading states

- Error handling

- Responsive design

- Demo data

- Presentation flow

35. What the Team Should Ask Before Every Feature

Whenever someone proposes a feature, ask:

Question 1

Does this directly improve detection, location, understanding, visibility or resolution of road hazards?

If no → probably postpone it.

Question 2

Does this help us prove the core product loop?

If no → P1/P2.

Question 3

Are we building a real capability or just making the demo look impressive?

This is especially important during a hackathon.

Question 4

Can this feature be removed without breaking the core system?

If yes → it is probably not core.

36. Core Product in One Sentence

The team should be able to explain the entire project in one sentence:

We turn vehicles into distributed road sensors, automatically detect and geolocate road hazards from dashcam footage, and convert that data into a public road-condition map and an accountability system for authorities.

37. Core Product in Three Layers

If you need an even shorter mental model:

Layer 1 — SEE

Vehicles observe roads.

Layer 2 — UNDERSTAND

AI converts footage into structured road-condition data.

Layer 3 — ACT

The public sees it and authorities can respond to it.

Everything else is an extension of these three layers.

38. Final Scope Boundary

MUST HAVE

Dashcam/video

↓

Pothole detection

↓

Location

↓

Database

↓

Map

↓

Road condition

↓

Authority visibility

SHOULD HAVE

Multiple observations

Duplicate detection

Severity

Route comparison

Status tracking

Public reporting

COULD HAVE

More hazard types

Mobile app

Live detection

Predictive maintenance

Citizen verification

Government integrations

Notifications

Advanced analytics

NOT NOW

Hardware manufacturing

Nationwide deployment

Every possible road hazard

Complex social features

Blockchain

Gamification

Full-scale government infrastructure

39. Final Product Definition

Product

Distributed Road Hazard Intelligence & Accountability Platform

Primary users

- Drivers / citizens

- Road authorities / municipalities

- Administrators / system operators

Primary data source

Vehicle dashcam footage + location data

Primary AI capability

Road-hazard detection

Primary data asset

Geospatial road-condition dataset

Primary public value

Safer and more informed route decisions

Primary government value

Evidence-based road maintenance prioritisation

Primary social value

Transparency and accountability

Core differentiator

Distributed vehicle-based road monitoring beyond traditionally monitored highways and major roads.

Core MVP

Capture → Detect → Geolocate → Map → Prioritise → Track

40. The North Star

The project should ultimately aim to answer four questions for every road:

What is the condition of this road?

How confident are we?

How long has the problem existed?

Has anything been done about it?

If your system can reliably answer those four questions, you have built something substantially bigger than a pothole detector.